## (a) Bit Stuffing

Bit stuffing is a technique used in HDLC to prevent the accidental occurrence of the flag sequence within a data frame.
In HDLC, each frame is delimited by a flag pattern 01111110. To ensure that this pattern does not appear inside the frame, the sender inserts a 0 after every sequence of five consecutive 1s in the transmitted bit stream.
The receiver removes the inserted 0 during reception.

## (b) HDLC Frame Construction Using Bit Stuffing

### Given

- Address = 11111100

- Control = 00110011

- Data = 11111000 11111100

- Frame Check Sequence (FCS) = 11001100 10101010

- HDLC Flag = 01111110

### Bit Stuffing Process

Address Field

11111100 → 111110100

(0 inserted after five consecutive 1s)

Control Field

00110011 → 00110011 (no stuffing required)

Data Field

11111000 → 111110000
11111100 → 111110100

FCS Field

11001100 10101010 → no stuffing required

### Final Bit-Stuffed Frame (Without Flags)

111110100 00110011 111110000 111110100 11001100 10101010

### Final HDLC Frame to be Transmitted

01111110 11111010000110011111100001111101001100110010101010 01111110

### Key Points (Exam Focus)

- Bit stuffing inserts a 0 after five consecutive 1s

- Stuffing is applied only between the flags

- Flags are not bit-stuffed

- Receiver removes the stuffed bits automatically